ANNEX II + III:
 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER
Contract title: Tender for boat with engine and interior  

Publication reference: HUSRB/1602/31/0209/T1 
Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee 
Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages: 

· Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer), 

· Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)  

· Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.
	1.

Item Number
	2.

Specifications Required
	3.

Specifications Offered
	4. 

Notes, remarks, 
ref to documentation
	5.

Evaluation Committee’s notes 

	
	Main goal of the contract is to supply the vessel/boat with engine and interior. Boat should have multiple tourist purposes, as stationary exhibition space and meeting space for cocktails and presentations. Boat also must be capable to navigate the Danube River over a long time. The capacity of the boat should be 12 passengers and 2 crew members. The boat must have side access with a ramp for people with disabilities. Length of the boat need to be between 9 and 12 m, with sufficient width for the required purposes. Speed of navigation from 10-14km/h.
The vessel/boat must be technically designed and built by register and certified company in Agency for business registration) in accordance with the applicable laws and building codes and procedures in the Republic of Serbia and according to the requirements of the Directorate for determination of the ability of vessels/boats for navigation of the Republic of Serbia.
Tenderer (supplier) also need to provide all technical documents (technical project, usage brochure, etc.) of the vessel/boat which is needed for the vessel/boat registration in Directorate for determination of the ability of vessels/boats for navigation of the Republic of Serbia.
MAIN CHARACTERISTICS:

Material: Fiberglass

Length over all: 10.80m

Width: 3.00 m

Draft: 0.55 m

Displacement: 5000 Kg

Fuel tank: 300 liters

Maximum speed: 14 km / h

Cruising speed: 10 km / h

Number of persons: 12 persons and captain with one crew member

TECHNICAL SPECIFICATION:

Hull of the ship: 

Construction: PU fillings, reinforced with glass, hull and fiberglass decks and other certified materials for use in shipbuilding.

Rubber bumper: Black and made of hard rubber

Color: White

Deck: 

Floor coverings and deck roads: Anti-slip surface made from biodegradable materials

Equipment: Fuel and stainless steel rolls, stainless steel bowls, bowstring (2x), stern (2x), stainless steel handles, A4 quality screws

Anchoring equipment: Anchor type Bruce 10 kilograms with belonging chain

Transom platform: Technical hole for access to engines

Zone for the back passengers' accommodation: Two benches with four seats

Cockpit: storage in the floor, side cabinets and bow anchorage for accommodation of anchors and other equipment

Dashboard:

Indicators: Number of rotations, battery voltage, speed

Switches: Waterproof switches

Steering wheel: Standard steering wheel dimensions min 350 mm

Controls: Separate controls, one for speed control, the other for transmission control

Electrical Components:
Batteries: Start - 12V (2x), 45 Ah, service - 12V deep cycle (2x), 220 Ah

Switchboard: 12V DC

Wiring: According to the ship's standard

Navigation equipment / lights: 

Navigation lights: Left and right LED (2x), mast and anchored assembly removable

Lighting of technical spaces and interior: LED lights (8x) technical space, LED lights (12x) interior,

Sound signalization: Electric siren

Interior / cab:
Floor coverings: Surfaces from biodegradable materials

Wall coverings: Fabric / canvas

Ceilings: Fabric / canvas, white

Seat coverings: Fabric / canvas

Windows: Polycarbonate glass

Doors: Sliding door for bow and stern entry

Seating schedule: Two sofas are centrally oriented to accommodate 8 people

Additionally: Kitchen with work desk sink and fridge / bathroom with washbasin

Standard Systems:
Bilge system: Three bilge pumps with automatic start, 1x bow thruster, 1x stern wagon, 1x interior space; one manual bilge pump with demo handle
Fuel system: Standard tanks of polyethylene 1x300 liters; 2x fuel filter / water separator with bypass due to redundancy; Fuel system hoses - Type A-1

Ventilation system: Exhaust fan mounted on the x2 bulkhead, fresh air supply in the indirect way

Electronic devices:
Radio station: Manual, portable

Radio / CD: Radio / CD player

Security equipment:
Anti - fire system: Two manual PP appliances and two automatic

Rescue equipment: Life jackets x14, first aid,

Motor / Engine:

Engine: Four stroke - double installation

	
	
	


15 January 2016
Page 1 of 6
c4f_annexiitechspeciiitechoffer_en.doc
15 January 2016
Page 2 of 6
c4f_annexiitechspeciiitechoffer_en.doc

